1.5W, 5kVRMS Complete Isolated DC-DC Converter

Features

- Complete Switch Mode Power Supply

  - High integration with internal transformer

  - Soft-start reduces input inrush current and output overshoot

  - Hiccup current-limit protection

  - Thermal shutdown

- 4.5 V to 5.5 V Input Voltage Range

- Selectable Output voltages

  - 3.3V and 5V output options

  - 3.7V and 5.4V output options provide headroom voltage to power LDO

- Delivers up to 1.5W(5V/300mA) Typical Output Power

- Excellent Load Transient Response

- Complies with CISPR32 Class B Radiated Emissions

- Up to ±3kV HBM and ±2kV CDM ESD protection

- Robust Galvanic Isolation Barrier

  - High lifetime: > 40 years

  - Up to 5kVRMS isolation rating

  - ±150 kV/μs typical CMTI

- Wide Operating Temperature Range: -40°C to 125°C

- Low Profile SOIC16-WB (10.30mm×7.50mm) Package

- Safety-Related Certifications

  - VDE certification according to DIN EN IEC 60747-17(VDE 0884-17):2021-10

  - UL certification according to UL 1577

  - CQC certification according to GB4943.1-2022

  - TUV certification:

  - according to EN 61010-1

  - according to EN 62368-1

Applications

- Instruments and Apparatuses

- Industrial automation

- Motor Control

- Medical Equipment

- Industrial Sensors

- Telecom Equipment

General Description

The CA-IS3115AW is a family of complete isolated DC-DC converters with up to 5kVRMS isolation rating. These devices integrate most of the components needed for an isolated power supply —switching controller, power switches, transformer, soft-start, protection circuit etc. — into a single, compact SOIC package. The result is an efficient and compact fully integrated solution that is easy to comply with EMI requirements and makes power-supply isolation design as easy as possible. Operating over an input voltage range of 4.5V to 5.5V, the CA-IS3115AW devices provide a fixed output voltage of 3.3V, 3.7V, 5V or 5.4V set by pin SEL. 3.7V and 5.4V output options provide headroom voltage to power an LDO. Only output, input bypass capacitors, and a pull-up resistor for 3.7V or 5.4V outputs are needed to finish the design.

The CA-IS3115AW devices feature a unique control scheme, which can quickly respond to load transient and accurately regulate the output voltage. The devices are capable of delivering a load up to 1.5W output power and offering soft-start, current limit, short-circuit protection and thermal shutdown protection features to better enhance the reliability of the system. The CA-IS3115AW devices include an enable input pin (EN). Connect the EN pin to the VINP input voltage or force this pin high to turn on the DC-DC converter. Force this pin low to disable the device and enter shutdown mode. In shutdown mode, the device stops switching operation with microampere standby supply current.

The CA-IS3115AW devices are available in wide-body SOIC16 package with creepage and clearance > 8mm, and operate over -40°C to +125°C temperature range.

Device Information

| Part number | Package | Package size (NOM) |
| --- | --- | --- |
| CA-IS3115AW | SOIC16-WB(W) | 10.30 mm × 7.50 mm |

Simplified Block Diagram

Ordering Information

Table 4-1. Ordering Information

| Part # | Output Power (W) | Isolation Rating(kVRMS) | Package |
| --- | --- | --- | --- |
| CA-IS3115AW | 1.5 | 5.0 | SOIC16-WB |

Table of Contents

1	Features	1

2	Applications	1

3	General Description	1

4	Ordering Information	2

5	Revision history	3

6	Pin Configuration and Description	4

7	Specifications	5

7.1	Absolute Maximum Ratings1, 2	5

7.2	ESD Ratings	5

7.3	Recommended Operating Conditions	5

7.4	Thermal Information	5

7.5	Power Ratings	5

7.6	Insulation Specifications	6

7.7	Safety-Related Certifications	7

7.8	Electrical Characteristics	8

7.9	MSL	8

7.10	Typical Operating Characteristics	9

8	Detailed Description	13

8.1	Overview	13

8.2	Output Voltage Selection	13

8.3	Protection Functions	14

8.3.1	UVLO and Soft-Start	14

8.3.2	Current-limit  Protection	14

8.3.3	Thermal Shutdown	14

9	Applications Information	15

9.1	Typical Application Circuit	15

9.2	Input and Output Capacitors	15

9.3	PCB Layout Guidelines	15

10	Package Information	17

11	Soldering Temperature (reflow) Profile	18

12	Tape and Reel Information	19

13	Important statement	20

Revision history

| Revision Number | Description | Revised Date | Page Changed |
| --- | --- | --- | --- |
| Preliminary Version | N/A | 2023/10/17 | N/A |
| Version 1.00 | Update isolation ratings | 2023/12/01 | 6,7 |
| Version 1.01 | Adds output derating curves | 2024/01/03 | 11,12 |
| Version 1.02 | Adds Short circuit protection function description | 2024/01/09 | 14 |
| Version 1.03 | Update VDE, UL, CQC, TUV information Update the test conditions of VIOSM | 2024/04/16 | 1,6,7 |
| Version 1.04 | Update the test conditions of VISO(LOAD) | 2024/05/14 | 8 |
| Version 1.05 | Add Capacitor value range in VISO pin in table of section 7.3 Update the Maximum Value of IVINP_SC Update Figure 9-2. Recommended Bypass Capacitors Placement | 2024/06/19 | 5,8,16 |
| Version 1.06 | Update VDE information： - Add Maximum impulse voltage VIMP - Update Maximum surge isolation voltage VIOSM Update CQC certification standards Add TUV certification information | 2024/09/05 | 1,6,7 |
| Version 1.07 | Update TUV information： Add TUV certification information | 2024/12/06 | 1,7 |
| Version 1.08 | Update the POD | 2026/06/15 | 19 |
| Version 1.09 | Update the efficiency curve of different Vin and Vout | 2026/06/15 | 11 |

Pin Configuration and Description

Figure 6-1. CA-IS3115AW Top View

Table 6-1. CA-IS3115AW Pin Description

| Pin name | Pin number | Type | Description |
| --- | --- | --- | --- |
| EN | 1 | Input | Enable input, active-high. Force this pin high to enable the device. Force this pin low to disable the device and put the device into shutdown mode. |
| VINP | 3 | Power | Primary side supply input. Connect VINP to GNDP with both 0.1μF and 10μF capacitors as close to the device(pin 3 and pin 4) as possible. |
| GNDP | 2, 4, 5, 6, 7, 8 | GND | Primary side local ground. |
| GNDS | 9, 10, 11, 12, 15, 16 | GND | Secondary side ground return connection for VISO. |
| SEL | 13 | Input | Output voltage VISO select pin: VISO = 5.0 V when SEL is shorted to VISO; VISO = 5.4 V when SEL is connected to VISO through a 100kΩ resistor; VISO = 3.3 V when SEL is shorted to GNDS; VISO = 3.7V when SEL is connected to GNDS through a 100kΩ resistor. Don’t leave SEL pin open. |
| VISO | 14 | Power | Isolated supply voltage pin. Connnect VISO to GNDS with both 0.1μF and 10μF capacitors as close to the device(pin 14 and pin 15) as possible. |

Specifications

Absolute Maximum Ratings1, 2

| Parameters | Minimum value |  | Maximum value | Unit |
| --- | --- | --- | --- | --- |
| VINP Power supply voltage | -0.5 | 6.0 |  | V |
| VISO Isolated supply voltage | -0.5 | 6.0 |  | V |
| EN EN input voltage | -0.5 | VINP+0.33 |  | V |
| SEL SEL input voltage | -0.5 | VISO+0.33 |  | V |
| TJ Junction temperature | -40 | 150 |  | °C |
| TSTG Storage temperature range | -65 | 150 |  | °C |
| Notes: - The stresses listed under “Absolute Maximum Ratings” are stress ratings only, not for functional operation condition. Exposure to absolute maximum rating conditions for extended periods may cause permanent damage to the device. - All voltage values are with respect to the local ground (GNDP or GNDS) and are peak voltage values. - Maximum voltage must not be exceed 6 V. |  |  |  |  |

ESD Ratings

|  |  | Value | Unit |
| --- | --- | --- | --- |
| VESD Electrostatic discharge | Human body model (HBM), per ANSI/ESDA/JEDEC JS-001, all pins1 | ±3000 | V |
|  | Charged device model (CDM), per JEDEC Specification JESD22-C101, all pins2 | ±2000 |  |
| Notes: - Per JEDEC document JEP155, 500V HBM allows safe manufacturing of standard ESD control process. - Per JEDEC document JEP157, 250V CDM allows safe manufacturing of standard ESD control process. |  |  |  |

Recommended Operating Conditions

| Parameters | Minimum value | Typical value | Maximum value | Unit |
| --- | --- | --- | --- | --- |
| VINP Power supply voltage | 4.5 | 5 | 5.5 | V |
| VEN EN input voltage | 0 |  | 5.5 | V |
| VISO Isolated supply voltage | 0 |  | 5.7 | V |
| VSEL SEL input voltage | 0 |  | 5.7 | V |
| CVISO Capacitor value range in VISO pin | 4.7 | 10 | 1000 | µF |
| TA Ambient Temperature | -40 |  | 125 | °C |
| TJ Junction temperature | -40 |  | 150 | °C |

Thermal Information

| Thermal Metric | CA-IS3115AW | Unit |
| --- | --- | --- |
|  | SOIC16-WB(W) |  |
| RθJA Junction-to-ambient thermal resistance | 68 | °C/W |
| RθJC Junction-to-case thermal resistance | 18 |  |

Power Ratings

| Parameters | Test Conditions | Minimum value | Typical value | Maximum value | Unit |
| --- | --- | --- | --- | --- | --- |
| PD Power dissipation | VINP = 5.5V, VISO = 5.4V, 300mA output current |  |  | 3 | W |

Insulation Specifications

| PARAMETR |  | TEST CONDITIONS | VALUE | UNIT |
| --- | --- | --- | --- | --- |
| CLR | External clearance1 | Shortest terminal-to-terminal distance through air | 8 | mm |
| CPG | External creepage1 | Shortest terminal-to-terminal distance across the package surface | 8 | mm |
| DTI | Distance through the insulation | Minimum internal gap (internal clearance) | 28 | μm |
| CTI | Comparative tracking index | DIN EN 60112 (VDE 0303-11); IEC 60112 | > 600 | V |
|  | Material group | According to IEC 60664-1 | I |  |
|  | Overvoltage category per IEC 60664-1 | Rated mains voltage ≤ 300VRMS | I-IV |  |
|  |  | Rated mains voltage ≤ 600VRMS | I-IV |  |
|  |  | Rated mains voltage ≤ 1000VRMS | I-III |  |
| DIN EN IEC 60747-17 (VDE 0884-17)2 |  |  |  |  |
| VIORM | Maximum repetitive peak isolation voltage | AC voltage (bipolar) | 2121 | VPK |
| VIOWM | Maximum working isolation voltage | AC voltage; Time dependent dielectric breakdown (TDDB) Test | 1500 | VRMS |
|  |  | DC voltage | 2121 | VDC |
| VIOTM | Maximum transient isolation voltage | VTEST = VIOTM, t = 60s (qualification); VTEST = 1.2 × VIOTM, t= 1s (100% production) | 7070 | VPK |
| VIMP | Maximum impulse voltage | 1.2/50-μs waveform per IEC 62368-1 | 9846 | VPK |
| VIOSM | Maximum surge isolation voltage3 | VIOSM ≥ 1.3 x VIMP; Tested in oil (qualification test), 1.2/50-μs waveform per IEC 62368-1 | 12800 | VPK |
| qpd | Apparent charge4 | Method a, After input/output safety test subgroup 2/3, Vini = VIOTM, tini = 60s; Vpd(m) = 1.2 × VIORM, tm = 10s | ≤ 5 | pC |
|  |  | Method a, After environmental tests subgroup 1, Vini = VIOTM, tini = 60s; Vpd(m) = 1.6 × VIORM, tm = 10s | ≤ 5 |  |
|  |  | Method b1, At routine test (100% production) and preconditioning (type test) Vini = 1.2 × VIOTM, tini = 1s; Vpd(m) = 1.875 × VIORM, tm = 1s | ≤ 5 |  |
| CIO | Barrier capacitance, input to output5 | VIO = 0.4 × sin(2πft), f = 1MHz | 3.5 | pF |
| RIO | Isolation resistance5 | VIO = 500V, TA = 25°C | > 1012 | Ω |
|  |  | VIO = 500V, 100°C ≤ TA ≤ 125°C | > 1011 |  |
|  |  | VIO = 500V at TS = 150°C | > 109 |  |
|  | Pollution degree |  | 2 |  |
| UL 1577 |  |  |  |  |
| VISO | Maximum withstanding isolation voltage | VTEST = VISO, t = 60s (qualification), VTEST = 1.2 × VISO, t = 1s (100% production) | 5000 | VRMS |
| NOTE: - Creepage and clearance requirements should be applied according to the specific equipment isolation standards of an application. Care should be taken to maintain the creepage and clearance distance of a board design to ensure that the mounting pads of the isolator on the printed-circuit board do not reduce this distance. Creepage and clearance on a printed-circuit board become equal in certain cases. Techniques such as inserting grooves and/or ribs on a printed circuit board are used to help increase these specifications. - This coupler is suitable for safe electrical insulation only within the safety ratings. Compliance with the safety ratings shall be ensured by means of suitable protective circuits. - Testing is carried out in air or oil to determine the intrinsic surge immunity of the isolation barrier. - Apparent charge is electrical discharge caused by a partial discharge (pd). - All pins on each side of the barrier tied together creating a two-terminal device. |  |  |  |  |

Safety-Related Certifications

| VDE | UL | CQC | TUV |
| --- | --- | --- | --- |
| Certified according to DIN EN IEC 60747-17(VDE 0884-17):2021-10; EN IEC 60747-17:2020+AC:2021 | Certified according to UL 1577 Component Recognition Program | Certified according to GB4943.1-2022 | Certified according to EN 61010-1 and EN 62368-1 |
| VIORM: 2121VPK VIOTM: 7070VPK VIOSM: 12800VPK | Single protection: 5000VRMS | Reinforced isolation (Altitude ≤ 5000m) | EN 61010-1 5000VRMS EN 62368-1 5000VRMS |
| Certification number: 40057278 (reinforced isolation) | Certification number: E511334 | Certification number: CQC23001406424 | Client reference number: 2253313 |

Electrical Characteristics

Over operating temperature range TA = -40 to 125°C, VINP = 4.5V to 5.5V, SEL connected to VISO, CVINP = CVISO = 10µF, unless otherwise specified. All typical specs are at TA = 25°C and VINP = 5V.

| Parameters |  | Test Conditions | Minimum value |  |  | TYP | Maximum value |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power Supply Input |  |  |  |  |  |  |  |  |  |
| IVINP_SD VINP shutdown current | EN = LOW |  |  |  | 0.5 |  | 30 |  | µA |
| IVINP_O VINP quiescent current IOUT = 0% load | EN = HIGH, SEL connected to VISO (5V output) |  |  |  | 4.9 |  | 20 |  | mA |
|  | EN = HIGH, SEL 100kΩ to VISO (5.4Voutput) |  |  |  | 5.2 |  | 20 |  | mA |
|  | EN = HIGH, SEL connected to GNDS (3.3V output) |  |  |  | 4.1 |  | 20 |  | mA |
|  | EN = HIGH, SEL 100kΩ to GNDS (3.7V output) |  |  |  | 4.3 |  | 20 |  | mA |
| IVINP_SC DC current from VINP supply under short circuit on VISO | VISO short to GNDS |  |  |  | 77 |  | 125 |  | mA |
| VUVLO+ Input undervoltage lockout rising threshold |  |  |  |  | 2.7 |  | 3 |  | V |
| VUVLO- Input undervoltage lockout falling threshold |  |  |  | 2.1 | 2.3 |  |  |  | V |
| VHYS(UVLO) Input undervoltage lockout hysteresis |  |  |  |  | 0.4 |  | 0.6 |  | V |
| EN, SEL pins |  |  |  |  |  |  |  |  |  |
| VIH_EN EN Input threshold, logic HIGH |  |  |  | 0.7VINP |  |  |  |  | V |
| VIL_EN EN Input threshold, logic LOW |  |  |  |  |  |  | 0.3VINP |  | V |
| IEN Input leakage current | VINP = 5V, VEN = 5V |  |  |  | 10 |  | 20 |  | µA |
| Isolated DC-DC Converter |  |  |  |  |  |  |  |  |  |
| VISO Isolated output voltage | SEL connected to VISO (5V output), IISO = 150mA |  |  | 4.75 | 5.0 |  | 5.25 |  | V |
|  | SEL 100KΩ to VISO (5.4V output), IISO = 150mA |  |  | 5.13 | 5.4 |  | 5.67 |  |  |
|  | SEL connected to GNDS (3.3V output), IISO = 200mA |  |  | 3.13 | 3.3 |  | 3.47 |  |  |
|  | SEL 100KΩ to GNDS (3.7V output), IISO = 200mA |  |  | 3.51 | 3.7 |  | 3.89 |  |  |
| ILOAD_MAX Maximum load currrent | SEL connected to VISO (5V output) |  |  | 240 | 300 |  |  |  | mA |
|  | SEL 100KΩ to VISO (5.4V output) |  |  | 240 | 300 |  |  |  |  |
|  | SEL connected to GNDS (3.3V output) |  |  | 320 | 400 |  |  |  |  |
|  | SEL 100KΩ to GNDS (3.7V output) |  |  | 320 | 400 |  |  |  |  |
| VISO(RIP) Voltage ripple on isolated supply output (pk-pk) | 20MHz bandwidth, SEL short to VISO (5V input, 5V or 5.4V output),IISO = 150mA |  |  |  | 65 |  |  |  | mV |
|  | 20MHz bandwidth, SEL short to GNDS (5V input, 3.3V or 3.7V output), IISO = 200mA |  |  |  | 55 |  |  |  |  |
| VISO(LINE) Line regulation | SEL short to VISO (5V or 5.4V output), IISO = 150mA, VINP = 4.5V to 5.5 V |  |  |  | 4 |  | 20 |  | mV/V |
|  | SEL short to GNDS (3.3V or 3.7V output), IISO = 200mA, VINP = 4.5V to 5.5V |  |  |  | 4 |  | 20 |  |  |
| VISO(LOAD) Load regulation | SEL short to VISO (5V input, 5V or 5.4V output), IISO = 0 to 240mA |  |  |  | 0.5% |  | 2% |  |  |
|  | SEL short to GNDS (5V input, 3.3V or 3.7V output), IISO = 0 to 320mA |  |  |  | 0.5% |  | 2% |  |  |
| EFF Efficiency@maximum load current | IISO = 300 mA, CLOAD = 0.1μF ¦¦ 10μF; VISO=5V, 5.4V |  |  |  | 60% |  |  |  |  |
|  | IISO = 300mA, CLOAD = 0.1μF ¦¦ 10μF; VISO=3.3V,3.7V |  |  |  | 50% |  |  |  |  |
| CMTI Common-mode transient immunity | Slew Rate of GNDP versus GNDS, VCM=1200VRMS |  |  | ±150 |  |  |  |  | kV/µs |
| tRISE VISO rise time | 10% to 90%, VISO =3.3V, 3.7V, 5.0V, 5.4V |  |  |  | 1 |  |  |  | ms |
| VISO ripple voltage (peak to peak) | 10% to 90% load step with 10mA/µs slew-rate; VISO ripple voltage difference at two loads |  |  |  | 100 |  |  |  | mV |
| VISO load transient response |  |  |  |  | 5 |  |  |  | µs |

MSL

| Parameters | Standard | Level |
| --- | --- | --- |

| MSL | IPC/JEDEC J-STD-020D.1 | MSL 3 |
| --- | --- | --- |

Typical Operating Characteristics

| Soft-start waveform: VINP =5V(yellow), VISO=5V(green), IISO=300mA(blue) | Soft-start waveform: VINP =5V(yellow), VISO=3.3V(green), IISO=300mA(blue) |
| --- | --- |
| Soft-start waveform: VINP =5V(yellow), VISO=5.4V(green), IISO=300mA(blue) | Soft-start waveform: VINP =5V(yellow), VISO=3.7V(green), IISO=300mA(blue) |
| VISO ripple voltage(peak to peak): VINP =5V, VISO=5V, IISO=300mA | VISO ripple voltage(peak to peak): VINP =5V, VISO=3.3V, IISO=300mA |
| VISO ripple voltage(peak to peak): VINP =5V, VISO=5.4V, IISO=300mA | VISO ripple voltage(peak to peak): VINP =5V, VISO=3.7V, IISO=300mA |
| VISO Load transient response: VINP =5V, VISO=5V Load-current: 30mA/270mA | VISO Load transient response: VINP =5V, VISO=3.3V Load-current: 30mA/270mA |
| VISO load regulation: VINP =5V, VISO=5V/5.4V/3.3V/3.7V Output current: 0mA-300mA | Power supply efficiency vs. temperature VINP =5V, VISO=5V/5.4V/3.3V/3.7V, output current = 300mA |
| Efficiency for 5V-5V/5V-3.3V/5V-5.4V/5V-3.7V @25°C | EN Pin input current vs. temperature |
| Line regulation: VINP =4.5v ~ 5.5V Output current: 300mA | VINP Pin input current vs. temperature (EN=0V, VINP=5V) |
| VINP=5V, output current derating vs. temperature | VINP=5.5V, output current derating vs. temperature |
| VINP=4.5V, output current derating vs. temperature |  |

Detailed Description

Overview

The CA-IS3115AW is a family of complete isolated DC-DC converters designed to provide isolated power with up to 1.5W output power across a 5kVRMS isolation barrier. The devices operate over 4.5V to 5.5V input voltage range and use a proprietary control mechanism. The input supply VINP is provided to the primary power controller that switches the power stage connected to the integrated transformer. Power is transferred to the secondary side and regulated to a fixed output voltage set by the SEL pin. The soft-start feature allows to reduce input inrush current and avoid output overshoot. These devices also incorporate an output enable (EN) control and undervoltage lockout(UVLO) function that allows the user to turn on the part at the desired input-voltage level. Figure 8-1 shows the CA-IS3115AW’s  functional block diagram. Connect the EN pin to VINP or force this pin high to turn on the DC-DC converter. Force this pin low to disable the device and enter low-current shutdown mode. These devices offer a ready-made, reliable, easy-to-use solution and allow users save PCB space and reduce design time for the popular 5V, 3.3V power supply systems.

These devices are provided with a robust overcurrent protection scheme that protects the devices under overload and output short-circuit conditions. A cycle-by-cycle peak current limit turns off the switching operation and triggers hiccup mode. Once the hiccup timeout period expires, soft-start is attempted again. Hiccup mode operation ensures low power dissipation under output short-circuit conditions.

Figure 8-1. Functional Block Diagrams

Output Voltage Selection

At startup, the CA-IS3115AW monitors the state of pin SEL after applying VINP supply voltage above the UVLO rising threshold or enabling via the EN pin, to build the desired regulation voltage level for the VISO output. See Table 8-1 for the output voltage selection.

Table 8-1. VISO Output Voltage Selection

| EN | SEL | VISO |
| --- | --- | --- |
| High | Sorted to VISO | 5V |
| High | 100kΩ to VISO | 5.4V |
| High | Shorted to GNDS | 3.3V |
| High | 100KΩ to GNDS | 3.7V |
| High | OPEN | Unsupported |
| Low | X | 0V |
| Note: Don’t leave SEL pin open. |  |  |

Protection Functions

UVLO and Soft-Start

The CA-IS3115AW undervoltage lockout (UVLO) on both VINP power supply and VISO isolated supply. Upon power-up, the primary side transformer driver is disabled while the VINP voltage is below the threshold voltage VUVLO+(2.7V, typ.), and VISO output is off. The output powers up once the threshold is met. This allows the user to turn on the part at stable input-voltage level.

For many applications, it is necessary to minimize the inrush current at start-up. The CA-IS3115AW devices built-in soft-start circuit significantly reduces the start-up current spike and output voltage overshoot. Once input power supply is applied at VINP pin, the internal soft-start circuit will control the power delivered to the output gradually increase, allowing for a graceful turn-on ramp.

Current-limit  Protection

The CA-IS3115AW devices are provided with an over-current protection scheme that protects the device under overload and output short-circuit conditions. A cycle-by-cycle peak current limit turns off the switching operation and triggers a hiccup mode whenever the switch current exceeds the internal current limit. Once the hiccup timeout period expires, soft-start is attempted again. The hiccup condition is cleared when the over-current is removed.

Thermal Shutdown

Thermal-shutdown protection limits total power dissipation in the device. When the junction temperature of the device exceeds 175°C(TSD), an on-chip thermal sensor shuts down the device, allowing the device to cool. The thermal sensor turns the device on again after the junction temperature cools and junction temperature drops to normal operation temperature range (< 150°C ) of the device.

Applications Information

Typical Application Circuit

The CA-IS3115AW devices are high-integration isolated power supply solution. Included in the package are the switching controller, power switches, transformer, and all support components. Only output and input bypass capacitors are needed to finish the design. For the applications with LDO post regulator, it needs a single 100kΩ external resistor to set the output level to 3.7V or 5.4V, see Figure 9-1 typical application circuit.

Figure 9-1. CA-IS3115AW Typical Application Circuit

Input and Output Capacitors

The input capacitors (between VINP and GNDP) are required to reduce the peak current drawn from input power source and reduce the switching noise, increase efficiency. For the input capacitors, choose the ceramic capacitor because they have the lowest equivalent series resistance (ESR), smallest size, and lowest cost. For most applications, we recommend to use at least 0.1μF and 10μF ceramic capacitors with X5R or X7R temperature characteristic.

The output capacitors between VISO and GNDS are required as well to keep the output voltage ripple small and to ensure loop stability. These bypass capacitors must have low impedance at the switching frequency. Ceramic capacitors are recommended due to their small size and low ESR. Make sure the capacitors do not degrade their capacitance significantly over temperature and DC bias. It is recommended to have at least 10μF of effective capacitance at output.

PCB Layout Guidelines

High switching frequencies and large peak currents make PCB layout a very important part of the isolated DC-DC converter design. Good PCB design minimizes excessive electromagnetic interference (EMI) and voltage gradients in the ground plane to avoid instability and regulation errors. Even with the high level of integration, like CA-IS3115AW, users may fail to achieve specified operation with a haphazard or poor layout. So careful PCB layout is critical to achieve clean and stable operation, and ensure that the grounding and heat sinking are acceptable.

Place the input capacitors, output capacitors, and the CA-IS3115AW IC on the same PCB layer. Place decoupling capacitors as close as possible to the CA-IS3115AW device pins, see Figure 9-2 recommended components placement for the PCB layout. The paths must be wide and short to minimize inductance, also any via holes must be avoided on these paths. Connect all of the ground (GNDP, GNDS) connections to as large a plane area as possible for best heat-sinking. To ensure isolation performance between the primary side and secondary side, on the top layer and bottom layer keep the space under the CA-IS3115AW device free from traces, vias, and pads to maintain maximum creepage distance (≥ 8mm).

Figure 9-2. Recommended Bypass Capacitors Placement

Package Information

The following figure illustrates the size drawing and recommended pad size of SOIC16-WB wide-body package for the CA-IS3115AW isolated DC-DC converter. All dimensions are in millimeters.

Soldering Temperature (reflow) Profile

Figure 11-1. Soldering Temperature (reflow) Profile

Table 11-1. Soldering Temperature Parameter

| Profile Feature | Pb-Free Soldering |
| --- | --- |
| Ramp-up rate (TL = 217°C to peak TP) | 3°C/s max |
| Time ts of preheat temp (Tsmin = 150°C to Tsmax = 200°C) | 60~120 seconds |
| Time tL to be maintained above 217°C | 60~150 seconds |
| Peak temperature TP | 260°C |
| Time tP within 5°C of actual peak temp | 30 seconds max |
| Ramp-down rate (peak TP to TL = 217°C) | 6°C/s max |
| Time from 25°C to peak temperature TP | 8 minutes max |

Tape and Reel Information

REEL DIMENSIONS

TAPE DIMENSIONS

| A0 | Dimension designed to accommodate the component width |
| --- | --- |
| B0 | Dimension designed to accommodate the component length |
| K0 | Dimension designed to accommodate the component thickness |
| W | Overall width of the carrier tape |
| P1 | Pitch between successive cavity centers |

QUADRANT ASSIGNMENTS FOR PIN 1 ORIENTATION IN TAPE

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA-IS3115AW | SOIC | W | 16 | 1000 | 330 | 16.4 | 10.9 | 10.7 | 3.2 | 12.0 | 16.0 | Q1 |

Important statement

The above information is for reference only and used for helping Chipanalog customers with design, research and development. Chipanalog reserves the rights to change the above information due to technological innovation without advance notice.

All Chipanalog products pass ex-factory test. As for specific practical applications, customers need to be responsible for evaluating and determining whether the products are applicable or not by themselves. Chipanalog's authorization for customers to use the resources are only limited to development of the related applications of the Chipanalog products. In addition to this, the resources cannot be copied or shown, and Chipanalog is not responsible for any claims, compensations, costs, losses, liabilities and the like arising from the use of the resources.

Trademark information

Chipanalog Inc.® and Chipanalog® are registered trademarks of Chipanalog.

http://www.chipanalog.com